EASM51.DOC

Instructions for using the EASM51 assembler

---------------------------------------------------------------------

Call: EASM51 filename options

Filename:  Name of the file to be assembled, without extension. The

assembler uses the file filename.A51 as the source file, the

file filename.LST as the list file, and the file filename.HEX

as the hexadecimal file.

Options:   -NOLIST   : do not generate listing

-NOHEX    : do not generate hex file

-NOSYMTAB : do not print symbol table in listing

The assembler leaves an exit code <>0 if an error has occured.

---------------------------------------------------------------------

Files:

EASM51.EXE : executable program

EASM51.PAS : Turbo-Pascal source code

EASM51.DOC : short documentation

---------------------------------------------------------------------

Line format:

A line genarally consists of three parts:

Label   Instruction   Comment

If the first character in a line is one of the letters A to Z, or a to z ,

the first symbol of the line is taken as a label.

Labels may end with a colon (:) -- this is however not strictly required.

Labels may be up to 8 characters long, and may contain the underscore

symbol (_). The assembler is case sensitive (i.e. upper case and lower case

characters are recognized).

If the first character is a space, the assembler decides that there is

no label, and continues.

The next part of the line is interpreted as an instruction (assembler-

pseudo- instruction or machine-language instruction). (Mnemonics)

When a semicolon (;) appears anywhere in the line, the text after it is

interpreted as comment.

Examples:

*********************************************************************

;

; 10 example lines for the EASM51 Assembler

;

ORG 0000H      ; start location

START    NOP    ; this is comment

NOP

MOV    A,#2    ; load constant

AGAIN LJMP   START   ; loop

; Comment only

;

**********************************************************************

Example file DEMO2.A51:

**********************************************************************

;       DEMO2.A51

;

ORG   1000H

START   MOV   A,R0

RET

TEXT    DB    '1234',0

;

END

*********************************************************************

The individual parts of a line may start at any column.

---------------------------------------------------------------------

Assembler Pseudo-Opcodes:

ORG    expr

DB     expr,expr,expr ...

DS     expr

Label  EQU    expr

END

The assembler uses an internal location counter, which marks the location

of the next byte to be stored in the program memory.

The instruction

ORG    expr

Assigns the value of the arithmetic expression to the location counter.

This instruction is used to make the program start at a specific location

in the memory (ORG: Origin).

The instruction

DS     expr

Increments the location counter with the value of the arithmetic experession.

It may also be used, by combination with ORG instructions, to manage the

internal RAM memory (DS: Define Storage).

The instruction

DB     expr,expr,....

Saves the values of the arithmetic expressions as bytes in the program memory,

and increments the location counter as required. String constants are also

allowed. (DB : Define Byte).

The instruction

Label  EQU    expr

Assigns the value of the arithmetic experession to Label. (EQU: Equate).

The instruction

END

marks the end of the source file.

---------------------------------------------------------------------

Notation of numeric and string constants:

numeric constants:

Decimal numbers:      normal notation.

Hexadecimal numbers:  marked by suffix H, must start with a number

0 to 9 to make them different from labels.

Binary numbers:       marked by suffix B.

Character constants:  a character enclosed between single quotes.

String constants:     a string of characters enclosed between quotes.

(only for use with DB instruction).

Examples:

Decimal numbers     0   00001   12324

Hexadecimal nos.    0H  1H      0ABCDH    01000H   0C000H

Binary numbers      1B  10B     11111010B

Character constants 'A'  '0'    'x'

String constants    'XYZ'       '0123456789H'      'Princess Hella'

---------------------------------------------------------------------

Arithmetic expressions:

Numerical constants and

Character constants are arithmetic expressions.

Labels              are arithmetic expressions.

$                   is an arithmetic expression,

value=value of the location counter.

When expr1 and expr2 are arithmetic expressions, then also:

-expr1         ; a negative value

expr1 + expr2  ; Sum

expr1 - expr2  ; Difference

expr1 * expr2  ; Product

expr1 / expr2  ; integer part of divide result

Note:

3+7*2 equals 3+(7*2) and gives 17.

---------------------------------------------------------------------

HEX-output

The assembler generates (writes) an object-code file that forms a

"HEX_DUMP". Each line of this file has the following format:

n1 n2:b1 b2   .....  bn n3

Where:

n1         Number of bytes to follow (as a 2-digit hexadecimal number).

n2         Address of bytes to follow (as a 4-digit hexadecimal number).

b1 bis bn  These are object-code bytes, each as a two-digit hexadecimal

number, separated by spaces.

n3         Sum of object-code bytes as a 4-digit hexadecimal number.

Example (DEMO2.HEX) :

07 1000:E8 22 31 32 33 34 00 01D4

---------------------------------------------------------------------

List file

The listing produced by the assembler has the following structure:

****** LISTING of EASM51 (DEMO2) ******

LINE LOC  OBJ        T        SOURCE

1 0000              ;       DEMO2.A51

2 0000              ;

3 0000                      ORG   1000H

4 1000 E8       [1] START   MOV   A,R0

5 1001 22       [2]         RET

6 1002 31 32 33     TEXT    DB    '1234',0

34 00

7 1007              ;

8 1007                      END

symboltable (2 symbols) :

START :1000      TEXT :1002

The column "LINE" contains the line number of the source text, in

decimal notation.

The column "LOC" (Location) indicates the value of the location (address)

counter (4 digit hexadecimal).

The column "OBJ" indicates the generated object code.

The column "T" shows the execution time of instructions in cycles. At a

clock of 12 MHz, one cycle lasts exactly one microsecond.

The column "SOURCE" shows the source code.

---------------------------------------------------------------------

Error reports of EASM51:

string length

Indicated text string is too long.

missing or invalid term

Missing valid arithmetic expression.

unknown expression

Arithmetic expression can not be computed.

missing expression

An arithmetic expression must follow.

missing term

Missing arithmetic term.

internal buffer overflow

Line generates too many OPCODE bytes

range error

Arithmetic expression does not fit in BYTE

syntax

SYNTAX error

unknown address

Indicated address (or label) is not known

relative jump range

Jump out of range

far jump range

Jump out of range

base value not bit addressable

as is.

unknown opcode

as is.

invalid expression

as is.

missing label

as is.

double defined label

as is.

invalid line

Line ending not allowed

--------------------------------------------------------------------------